VX-3R Specifications           May, 2007

Frequency Ranges:					RX 	0.5-1.8 MHz (AM Broadcast)

(USA Version)														1.8-30 MHz	 (SW Band)

30-76 MHz (50 MHz HAM)

76-108 MHz (FM)

108-137 MHz (Air Band)

137-174 MHz (144 MHz HAM)

174-222 MHz (VHF TV)

222-420 MHz (ACT1)

420-470 MHz (430 MHz HAM)

470-800(729) MHz (UHF TV)

(757-774) MHz (UHF TV)

800-999 MHz (GEN2; USA Cellular Blocked)

TX	144-146(148) MHz

430-440(450) MHz

Channel Steps:									5/9/8.33/10/12.5/15/20/25/50/100 kHz

Frequency Stability:						±5 ppm (–10 °C to +60 °C)

Repeater Shift:									±600 kHz (144 MHz)

±1.6/5.0/7.6 MHz (430 MHz)

Emission Type:									F2D , F3E

Antenna Impedance:						50 

Supply Voltage:									Nominal:	3.7 V DC,	Negative Ground

Operating:	3.5 ~ 7 V,	Negative Ground (EXT DC Jack)

5.0 ~ 7 V,	Negative Ground (EXT DC Jack with Charging)

Current Consumption:				120 mA (Receive)

60 mA (Standby, Saver Off)

30 mA (Standby, Saver On, Save Ratio 1:2)

50 mA (Radio Band Receive)

100 µA (Auto Power Off)

1.3 A (1.5 W Tx , 144 MHz) 3.7 V DC

1.6 A (3 W Tx , 144 MHz) 6 V DC

1.2 A (1 W Tx , 430 MHz) 3.7 V DC

1.8 A (2 W Tx , 430 MHz) 6 V DC

Operating Temperature:			–20 °C to +60 °C

Case Size (W x H x D):				1.9” x 3.2” x 0.9” (47 x 81 x 23 mm) (W/O knob & antenna)

Weight:														4.6 oz (130 g) With FNB-82LI & antenna

Transmitter

RF Power Output:							1.5 W (@ 4.5 V AA x 3 or 3.7 V FNB-82LI  144 MHz)

3 W (@ 6 V or EXT DC  144 MHz)

1 W (@4.5 V AA x 3 or 3.7 V FNB-82LI  430 MHz)

2 W (@ 6 V or EXT DC  430 MHz)

Low 0.1 W (@ 4.5 V AA x 3 or 3.7 V FNB-82LI)

Low 0.3 W (@ 6 V or EXT DC)

Modulation Type:							Variable Reactance F2D , F3E

Maximum Deviation:					±5 kHz (F2D , F3E)

Spurious Emission:						At least 60 dB below (HIGH)

At least 50 dB below (LOW or less than 1 W)

Microphone Impedance:			2 k

Receiver

Circuit Type:										AM, NFM: Double-Conversion Superheterodyne

WFM: Triple-Conversion Superheterodyne

AM Radio/FM Radio: Single-Conversion Superheterodyne

Intermediate Frequencies:		1st: 47.25 MHz (AM, NFM)

1st: 45.8 MHz (WFM)

1st: 130 kHz (AM Radio/FM Radio)

2nd: 450 kHz (AM, NFM)

2nd: 10.7 MHz (WFM)

3rd: 1 MHz (WFM)

Sensitivity:												3 µV for 10 dB SN (0.5-1.8 MHz, AM Radio)

3 µV for 10 dB SN (0.5-30 MHz, AM)

0.35 µV TYP for 12 dB SINAD (30-54 MHz, NFM)

1 µV TYP for 12 dB SINAD (54-76 MHz, NFM)

1.5 µV TYP for 12 dB SINAD (76-108 MHz, FM Radio)

1.5 µV TYP for 10 dB SN (108-137 MHz, AM)

0.2 µV for 12 dB SINAD (137-140 MHz, NFM)

0.16 µV for 12 dB SINAD (140-150 MHz, NFM)

0.2 µV for 12 dB SINAD (150-174 MHz, NFM)

1 µV TYP for 12 dB SINAD (174-225 MHz, NFM)

0.5 µV for 12 dB SINAD (300-350 MHz, NFM)

0.2 µV for 12 dB SINAD (350-400 MHz, NFM)

0.18 µV for 12 dB SINAD (400-470 MHz, NFM)

1.5 µV for 12 dB SINAD (470-540 MHz, WFM)

3 µV TYP for 12 dB SINAD (540-800 MHz, WFM)

1.5 µV TYP for 12 dB SINAD (800-999 MHz, NFM)

USA Version Cellular Blocked

Selectivity:												NFM, AM:	12 kHz/35 kHz (–6 dB /–60 dB)

WFM	:		200 kHz / 300 kHz (–6 dB/–20 dB)

AF Output:												50 mW @ 8  for 10 % THD (@ 3.7 V)

100 mW @8  for 10 % THD (@ 6 V)